Tennessee Eastman Process (TEP) - Control Structure Analysis

** Summary**

The Fortran implementation contains:

11 Manipulated Variables (XMV) - Valve positions and control outputs

41 Process Measurements (XMEAS) - Sensor readings

20 Disturbance Variables (IDV) - Fault injection points

20 Control Setpoints (SETPT) - Controller targets

Multiple Control Loops - Including cascade, feedback, and feedforward control

**1. MANIPULATED VARIABLES (XMV) - 11 Total**

Control outputs (valve positions, flow rates) that operators can adjust:

| XMV # | Description | Unit | Range | Default | Control Type |
| --- | --- | --- | --- | --- | --- |
| **XMV(1)** | D Feed Flow (stream 2) | % | 0-100 | 63.0% | Continuous valve |
| **XMV(2)** | E Feed Flow (stream 3) | % | 0-100 | 53.0% | Continuous valve |
| **XMV(3)** | A Feed Flow (stream 1) | % | 0-100 | 24.0% | Continuous valve |
| **XMV(4)** | A and C Feed Flow (stream 4) | % | 0-100 | 61.0% | Continuous valve |
| **XMV(5)** | Compressor Recycle Valve | % | 0-100 | 22.0% | Continuous valve |
| **XMV(6)** | Purge Valve (stream 9) | % | 0-100 | 40.0% | Continuous valve |
| **XMV(7)** | Separator Pot Liquid Flow (stream 10) | % | 0-100 | 38.0% | Continuous valve |
| **XMV(8)** | Stripper Liquid Product Flow (stream 11) | % | 0-100 | 46.0% | Continuous valve |
| **XMV(9)** | Stripper Steam Valve | % | 0-100 | 47.0% | Continuous valve |
| **XMV(10)** | Reactor Cooling Water Flow | % | 0-100 | 41.0% | Continuous valve |
| **XMV(11)** | Condenser Cooling Water Flow | % | 0-100 | 18.0% | Continuous valve |

Note: All XMV values are continuous (0.0-100.0%)

**2. PROCESS MEASUREMENTS (XMEAS) - 41 Total**

Sensor readings:

**2.1 Flow Measurements (10 variables)**

| XMEAS # | Description | Unit | Typical Range |
| --- | --- | --- | --- |
| XMEAS(1) | A Feed (stream 1) | kscmh | 0-15 |
| XMEAS(2) | D Feed (stream 2) | kg/h | 2000-4000 |
| XMEAS(3) | E Feed (stream 3) | kg/h | 3000-5000 |
| XMEAS(4) | A and C Feed (stream 4) | kscmh | 10-25 |
| XMEAS(5) | Recycle Flow (stream 8) | kscmh | 20-35 |
| XMEAS(6) | Reactor Feed Rate (stream 6) | kscmh | 35-50 |
| XMEAS(10) | Purge Rate (stream 9) | kscmh | 0.1-2.0 |
| XMEAS(14) | Prod Sep Underflow (stream 10) | m³/h | 20-30 |
| XMEAS(17) | Stripper Underflow (stream 11) | m³/h | 15-25 |
| XMEAS(19) | Stripper Steam Flow | kg/h | 200-400 |

**2.2 Pressure Measurements (3 variables)**

| XMEAS # | Description | Unit | Typical Range |
| --- | --- | --- | --- |
| XMEAS(7) | Reactor Pressure | kPa | 2700-2900 |
| XMEAS(13) | Prod Sep Pressure | kPa | 2600-2800 |
| XMEAS(16) | Stripper Pressure | kPa | 3000-3500 |

**2.3 Level Measurements (3 variables)**

| XMEAS # | Description | Unit | Typical Range |
| --- | --- | --- | --- |
| XMEAS(8) | Reactor Level | % | 40-70 |
| XMEAS(12) | Product Sep Level | % | 40-70 |
| XMEAS(15) | Stripper Level | % | 40-70 |

**2.4 Temperature Measurements (4 variables)**

| XMEAS # | Description | Unit | Typical Range |
| --- | --- | --- | --- |
| XMEAS(9) | Reactor Temperature | °C | 120-135 |
| XMEAS(11) | Product Sep Temp | °C | 70-90 |
| XMEAS(18) | Stripper Temperature | °C | 60-80 |
| XMEAS(21) | Reactor Cooling Water Outlet Temp | °C | 85-100 |
| XMEAS(22) | Separator Cooling Water Outlet Temp | °C | 70-85 |

**2.5 Composition Measurements (19 variables)**

Stream 6 (Reactor Feed) - XMEAS(23-28):

Component A, B, C, D, E, F (mole %)

Stream 9 (Purge) - XMEAS(29-36):

Component A, B, C, D, E, F, G, H (mole %)

Stream 11 (Stripper Product) - XMEAS(37-41):

Component D, E, F, G, H (mole %)

**2.6 Other Measurements**

| XMEAS # | Description | Unit | Typical Range |
| --- | --- | --- | --- |
| XMEAS(20) | Compressor Work | kW | 200-300 |

**3. CONTROL LOOPS STRUCTURE**

Based on the Fortran code analysis, the TEP contains multiple control loops:

**3.1 Primary Control Loops **

| Loop # | Controller | Controlled Variable (CV) | Manipulated Variable (MV) | Setpoint | Type |
| --- | --- | --- | --- | --- | --- |
| **1** | Production Rate | Reactor Feed Rate (XMEAS 6) | A Feed Flow (XMV 3) | Variable | Flow Control |
| **2** | Production Rate | Reactor Feed Rate (XMEAS 6) | D Feed Flow (XMV 1) | Variable | Flow Control |
| **3** | Reactor Pressure | Reactor Pressure (XMEAS 7) | Purge Valve (XMV 6) | ~2800 kPa | Pressure Control |
| **4** | Reactor Level | Reactor Level (XMEAS 8) | Compressor Recycle (XMV 5) | ~50% | Level Control |
| **5** | Reactor Temperature | Reactor Temp (XMEAS 9) | Reactor Cooling Water (XMV 10) | ~120°C | Temperature Control |
| **6** | Separator Pressure | Sep Pressure (XMEAS 13) | Compressor Recycle (XMV 5) | ~2700 kPa | Pressure Control |
| **7** | Separator Level | Sep Level (XMEAS 12) | Sep Liquid Flow (XMV 7) | SETPT(7) = 50% | **Level Control** |
| **8** | Stripper Level | Stripper Level (XMEAS 15) | Stripper Product Flow (XMV 8) | SETPT(8) = 50% | **Level Control** |
| **9** | Stripper Temperature | Stripper Temp (XMEAS 18) | Stripper Steam (XMV 9) | ~80°C | Temperature Control |
| **10** | Reactor Cooling | Cooling Water Outlet (XMEAS 21) | Reactor Cooling Water (XMV 10) | SETPT(10) = 94.6°C | **Temperature Control** |
| **11** | Condenser Cooling | Condenser Cooling | Condenser Cooling Water (XMV 11) | Variable | Temperature Control |

Total Confirmed Control Loops: 11 primary loops

**4. CASCADE CONTROL STRUCTURES**

**4.1 Reactor Temperature Cascade Control**

Outer Loop (Primary):

CV: Reactor Temperature (XMEAS 9)

SP: ~120-135°C

Output: Setpoint for inner loop

Inner Loop (Secondary):

CV: Reactor Cooling Water Outlet Temperature (XMEAS 21)

SP: SETPT(10) = 94.6°C (from outer loop)

MV: Reactor Cooling Water Flow (XMV 10)

Purpose: Provides faster disturbance rejection and prevents cooling water valve saturation.

**4.2 Production Rate Cascade **

Outer Loop:

CV: Product composition or production rate

SP: Operator-defined production target

Output: Feed flow setpoints

Inner Loops:

CV: Individual feed flows (XMEAS 1, 2, 3, 4)

MV: Feed valves (XMV 1, 2, 3, 4)

**5. P&ID CONTROL ELEMENTS**

**5.1 Major Equipment Units**

Reactor (CSTR - Continuous Stirred Tank Reactor)

- Pressure control (PC)

- Level control (LC)

- Temperature control (TC - cascade)

- Agitator speed control

Product Separator (Flash Drum)

- Pressure control (PC)

- Level control (LC)

- Temperature indication (TI)

Stripper Column (Distillation)

- Level control (LC)

- Temperature control (TC)

- Pressure indication (PI)

- Steam flow control (FC)

Compressor

- Work/power monitoring

- Recycle flow control

Condenser

- Cooling water flow control (FC)

- Temperature control (TC)

**5.2 Control Instrumentation**

| Symbol | Count | Description |
| --- | --- | --- |
| **FC** | 11 | Flow Controllers (all XMV variables) |
| **LC** | 3 | Level Controllers (Reactor, Separator, Stripper) |
| **PC** | 2 | Pressure Controllers (Reactor, Separator) |
| **TC** | 4 | Temperature Controllers (Reactor, Separator, Stripper, Cooling) |
| **FT** | 10 | Flow Transmitters |
| **LT** | 3 | Level Transmitters |
| **PT** | 3 | Pressure Transmitters |
| **TT** | 5 | Temperature Transmitters |
| **AT** | 19 | Analyzers (Composition measurements) |

Total Control Loops: 11 controllers

Total Measurements: 41 transmitters

Overall: 52 Features

**6. CONTROL SETPOINTS (SETPT Array)**

The Fortran code contains a SETPT array with 20 elements:

| SETPT # | Description | Default Value | Unit | Associated Controller |
| --- | --- | --- | --- | --- |
| SETPT(1-6) | Reserved/Unused | 0.0 | - | - |
| **SETPT(7)** | **Product Separator Level** | **50.0** | **%** | **LC-7** |
| **SETPT(8)** | **Stripper Level** | **50.0** | **%** | **LC-8** |
| SETPT(9) | Reserved/Unused | 0.0 | - | - |
| **SETPT(10)** | **Reactor Cooling Water Temp** | **94.6** | **°C** | **TC-10 (Cascade Inner)** |
| SETPT(11-20) | Reserved/Unused | 0.0 | - | - |

Note: It looks like the Fortran code has 20 setpoint slots, but only 3 are actively used in the base control configuration.

**7. KEY OPERATING PARAMETERS**

**7.1 Normal Operating Conditions**

| Parameter | Normal Range | Optimal Value | Critical Limits |
| --- | --- | --- | --- |
| **Reactor Temperature** | 120-135°C | 125°C | Max: 140°C |
| **Reactor Pressure** | 2700-2900 kPa | 2800 kPa | Max: 3000 kPa |
| **Reactor Level** | 40-70% | 50% | Min: 30%, Max: 80% |
| **Production Rate** | 7000-9000 kg/h | 8000 kg/h | - |
| **Purge Rate** | 0.1-2.0 kscmh | 0.5 kscmh | Max: 3.0 kscmh |
| **Stripper Steam** | 200-400 kg/h | 300 kg/h | Max: 500 kg/h |
| **Cooling Water Temp** | 85-100°C | 94.6°C | Max: 105°C |

**7.2 Feed Composition Ranges**

Stream 4 (A and C Feed):

Component A: 30-50 mole %

Component C: 50-70 mole %

Stream 2 (D Feed):

Pure D component

Stream 3 (E Feed):

Pure E component

**8. SUMMARY STATISTICS**

| Category | Count | Details |
| --- | --- | --- |
| **Total Control Loops** | **11** | All feedback controllers |
| **Cascade Control Loops** | **1-2** | Reactor temperature (confirmed), Production rate (inferred) |
| **Manipulated Variables** | **11** | XMV(1) to XMV(11) |
| **Process Measurements** | **41** | XMEAS(1) to XMEAS(41) |
| **Setpoints** | **20 slots** | Only 3 actively used (SETPT 7, 8, 10) |
| **Disturbance Variables** | **20** | IDV(1) to IDV(20) |
| **Level Controllers** | **3** | Reactor, Separator, Stripper |
| **Temperature Controllers** | **4** | Reactor, Separator, Stripper, Cooling |
| **Pressure Controllers** | **2** | Reactor, Separator |
| **Flow Controllers** | **11** | All XMV outputs |