TEP Complete Technical Reference for Lanny

Table of Contents

[TEP Process Overview](#1-tep-process-overview)

[Control Structure](#2-control-structure)

[Process Variables and Parameters](#3-process-variables-and-parameters)

[How Parameters Are Calculated](#4-how-parameters-are-calculated)

[Plant Control Strategy](#5-plant-control-strategy)

[Anomaly Detection System](#6-anomaly-detection-system)

[System Architecture](#7-system-architecture)

[Quick Reference Tables](#8-quick-reference-tables)

1. TEP Process Overview

1.1 What is TEP?

The Tennessee Eastman Process (TEP) is a realistic industrial chemical plant simulation originally developed by Eastman Chemical Company in 1993. It simulates a complete chemical production process with:

5 Major Equipment Units: Reactor, Separator, Compressor, Condenser, Stripper

8 Chemical Components: A, B, C, D, E (reactants), F (byproduct), G, H (products)

52 Process Variables: 41 measurements + 11 manipulated variables

20 Fault Types: Realistic industrial disturbances (IDV_1 to IDV_20)

1.2 Chemical Reactions

The TEP simulates the following reactions:

A(g) + C(g) + D(g) → G(liq)    (Product G)
A(g) + C(g) + E(g) → H(liq)    (Product H)
A(g) + E(g) → F(liq)           (Byproduct F)
3D(g) → 2F(liq)                (Side reaction)

Key Points:

Reactions occur in a CSTR (Continuous Stirred Tank Reactor)

Exothermic reactions (release heat) → requires cooling

Gas-phase reactants → Liquid-phase products

Byproduct F is undesirable but unavoidable

1.3 Process Flow

Feed Streams (A, D, E, A+C)
    ↓
Reactor (CSTR) → Exothermic reactions
    ↓
Product Separator (Flash) → Separate gas/liquid
    ↓ (gas)              ↓ (liquid)
Compressor           Stripper Column
    ↓                    ↓
Condenser            Product (G+H)
    ↓
Recycle to Reactor

2. Control Structure

2.1 Total Control Loops: **11 Controllers**

| Loop # | Controller Type | Controlled Variable (CV) | Manipulated Variable (MV) | Setpoint |
| --- | --- | --- | --- | --- |
| **1** | Flow Control (FC) | A Feed Flow (XMEAS 1) | A Feed Valve (XMV 3) | Variable |
| **2** | Flow Control (FC) | D Feed Flow (XMEAS 2) | D Feed Valve (XMV 1) | Variable |
| **3** | Pressure Control (PC) | Reactor Pressure (XMEAS 7) | Purge Valve (XMV 6) | ~2800 kPa |
| **4** | Level Control (LC) | Reactor Level (XMEAS 8) | Compressor Recycle (XMV 5) | ~50% |
| **5** | Temperature Control (TC) | Reactor Temp (XMEAS 9) | Reactor Cooling Water (XMV 10) | ~120°C |
| **6** | Pressure Control (PC) | Separator Pressure (XMEAS 13) | Compressor Recycle (XMV 5) | ~2700 kPa |
| **7** | Level Control (LC) | Separator Level (XMEAS 12) | Sep Liquid Flow (XMV 7) | **SETPT(7) = 50%** |
| **8** | Level Control (LC) | Stripper Level (XMEAS 15) | Stripper Product Flow (XMV 8) | **SETPT(8) = 50%** |
| **9** | Temperature Control (TC) | Stripper Temp (XMEAS 18) | Stripper Steam (XMV 9) | ~80°C |
| **10** | Temperature Control (TC) | Cooling Water Outlet (XMEAS 21) | Reactor Cooling Water (XMV 10) | **SETPT(10) = 94.6°C** |
| **11** | Flow Control (FC) | Condenser Cooling | Condenser Cooling Water (XMV 11) | Variable |

2.2 Cascade Control Structure

Reactor Temperature Cascade Control (Most Important):

┌─────────────────────────────────────────┐
│ OUTER LOOP (Primary Controller)        │
│ CV: Reactor Temperature (XMEAS 9)      │
│ SP: 120-135°C                           │
│ Output: Setpoint for inner loop        │
└──────────────┬──────────────────────────┘
               ↓
┌─────────────────────────────────────────┐
│ INNER LOOP (Secondary Controller)      │
│ CV: Cooling Water Outlet (XMEAS 21)    │
│ SP: SETPT(10) = 94.6°C (from outer)    │
│ MV: Reactor Cooling Water Flow (XMV 10)│
└─────────────────────────────────────────┘

2.3 P&ID Instrumentation Summary

| Instrument Type | Count | Description |
| --- | --- | --- |
| **FC** (Flow Controllers) | 11 | All XMV variables control flows |
| **LC** (Level Controllers) | 3 | Reactor, Separator, Stripper |
| **PC** (Pressure Controllers) | 2 | Reactor, Separator |
| **TC** (Temperature Controllers) | 4 | Reactor, Separator, Stripper, Cooling |
| **FT** (Flow Transmitters) | 10 | Measure flow rates |
| **LT** (Level Transmitters) | 3 | Measure liquid levels |
| **PT** (Pressure Transmitters) | 3 | Measure pressures |
| **TT** (Temperature Transmitters) | 5 | Measure temperatures |
| **AT** (Analyzers) | 19 | Composition measurements |

Total: 11 controllers, 41 transmitters

3. Process Variables and Parameters

3.1 Manipulated Variables (XMV) - 11 Total

| XMV # | Description | Unit | Range | Default | Physical Meaning |
| --- | --- | --- | --- | --- | --- |
| **XMV(1)** | D Feed Flow (stream 2) | % | 0-100 | 63.0% | Valve position controlling D input |
| **XMV(2)** | E Feed Flow (stream 3) | % | 0-100 | 53.0% | Valve position controlling E input |
| **XMV(3)** | A Feed Flow (stream 1) | % | 0-100 | 24.0% | Valve position controlling A input |
| **XMV(4)** | A and C Feed Flow (stream 4) | % | 0-100 | 61.0% | Valve position controlling A+C input |
| **XMV(5)** | Compressor Recycle Valve | % | 0-100 | 22.0% | Recycle loop control valve |
| **XMV(6)** | Purge Valve (stream 9) | % | 0-100 | 40.0% | Waste removal valve |
| **XMV(7)** | Separator Pot Liquid Flow (stream 10) | % | 0-100 | 38.0% | Separator bottom outlet valve |
| **XMV(8)** | Stripper Liquid Product Flow (stream 11) | % | 0-100 | 46.0% | Stripper product outlet valve |
| **XMV(9)** | Stripper Steam Valve | % | 0-100 | 47.0% | Steam heating control valve |
| **XMV(10)** | Reactor Cooling Water Flow | % | 0-100 | 41.0% | Reactor cooling valve |
| **XMV(11)** | Condenser Cooling Water Flow | % | 0-100 | 18.0% | Condenser cooling valve |

Notes:

All XMV values are continuous (0.0-100.0%), not discrete

Theoretically, changing XMV values affects process dynamics; however, in reality, seems not very responsible.

3.2 Process Measurements (XMEAS) - 41 Total

#### **Group 1: Flow Measurements (10 variables)**

| XMEAS # | Description | Unit | Normal Range | Critical Limits |
| --- | --- | --- | --- | --- |
| XMEAS(1) | A Feed (stream 1) | kscmh | 0.15-0.35 | Max: 0.5 |
| XMEAS(2) | D Feed (stream 2) | kg/h | 3500-3800 | Max: 4500 |
| XMEAS(3) | E Feed (stream 3) | kg/h | 4300-4700 | Max: 5500 |
| XMEAS(4) | A and C Feed (stream 4) | kscmh | 8.5-10.0 | Max: 12.0 |
| XMEAS(5) | Recycle Flow (stream 8) | kscmh | 25-29 | Max: 35 |
| XMEAS(6) | Reactor Feed Rate (stream 6) | kscmh | 40-45 | Max: 50 |
| XMEAS(10) | Purge Rate (stream 9) | kscmh | 0.1-2.0 | Max: 3.0 |
| XMEAS(14) | Prod Sep Underflow (stream 10) | m³/h | 20-30 | Max: 35 |
| XMEAS(17) | Stripper Underflow (stream 11) | m³/h | 15-25 | Max: 30 |
| XMEAS(19) | Stripper Steam Flow | kg/h | 200-400 | Max: 500 |

#### **Group 2: Pressure Measurements (3 variables)**

| XMEAS # | Description | Unit | Normal Range | Critical Limits |
| --- | --- | --- | --- | --- |
| XMEAS(7) | Reactor Pressure | kPa | 2700-2900 | **Max: 3000 kPa** |
| XMEAS(13) | Product Sep Pressure | kPa | 2600-2800 | Max: 2900 |
| XMEAS(16) | Stripper Pressure | kPa | 3000-3500 | Max: 3600 |

Safety Critical: Reactor pressure must stay below 3000 kPa to prevent vessel rupture!

#### **Group 3: Level Measurements (3 variables)**

| XMEAS # | Description | Unit | Normal Range | Critical Limits |
| --- | --- | --- | --- | --- |
| XMEAS(8) | Reactor Level | % | 40-70 | Min: 30%, Max: 80% |
| XMEAS(12) | Product Sep Level | % | 40-70 | Min: 30%, Max: 80% |
| XMEAS(15) | Stripper Level | % | 40-70 | Min: 30%, Max: 80% |

Safety Critical: Levels below 30% risk pump cavitation; above 80% risk overflow

#### **Group 4: Temperature Measurements (5 variables)**

| XMEAS # | Description | Unit | Normal Range | Critical Limits |
| --- | --- | --- | --- | --- |
| XMEAS(9) | Reactor Temperature | °C | 120-135 | **Max: 140°C** |
| XMEAS(11) | Product Sep Temp | °C | 70-90 | Max: 100 |
| XMEAS(18) | Stripper Temperature | °C | 60-80 | Max: 90 |
| XMEAS(21) | Reactor Cooling Water Outlet Temp | °C | 85-100 | Max: 105 |
| XMEAS(22) | Separator Cooling Water Outlet Temp | °C | 70-85 | Max: 95 |

Safety Critical: Reactor temperature above 140°C risks thermal runaway!

#### **Group 5: Composition Measurements (19 variables)**

Stream 6 (Reactor Feed) - XMEAS(23-28): Components A, B, C, D, E, F (6 variables)

Stream 9 (Purge) - XMEAS(29-36): Components A, B, C, D, E, F, G, H (8 variables)

Stream 11 (Stripper Product) - XMEAS(37-41): Components D, E, F, G, H (5 variables)

Total: 6 + 8 + 5 = 19 composition measurements

#### **Group 6: Other Measurements**

| XMEAS # | Description | Unit | Normal Range |
| --- | --- | --- | --- |
| XMEAS(20) | Compressor Work | kW | 200-300 |

4. How Parameters Are Calculated

4.1 Fortran Simulation Engine

The TEP simulation uses compiled Fortran code (temain_mod.so) that implements:

- Mass Balance Equations: Conservation of mass for each component

- Energy Balance Equations: Heat generation/removal calculations

- Reaction Kinetics: Arrhenius equations for reaction rates

- Thermodynamic Properties: Vapor-liquid equilibrium (VLE)

- Equipment Models: Reactor, separator, compressor, stripper dynamics

Controller Tuning (inferred from stable operation):

Level controllers: Slow integral action (Ti = 10-20 min)

Pressure controllers: Moderate gain (Kc = 1-5)

Temperature controllers: PID with derivative filtering (Td = 1-5 min)

Flow controllers: Fast PI response (Ti = 1-3 min)

4.4 Anomaly Detection Calculation

# From backend/app.py

# Step 1: Collect 52 features (XMEAS 1-41 + XMV 1-11)
features = [XMEAS_1, XMEAS_2, ..., XMEAS_41, XMV_1, ..., XMV_11]

# Step 2: Standardize features (zero mean, unit variance)
features_scaled = (features - mean) / std

# Step 3: Project onto principal components
scores = features_scaled @ principal_components

# Step 4: Calculate T² statistic (Hotelling's T²)
T2 = scores @ inv(covariance) @ scores.T

# Step 5: Compare to threshold
if T2 > threshold:
    anomaly_detected = True

Threshold: Default = 0.055 (calibrated from normal operation data)

5. Plant Control Strategy

5.1 Control Objectives

| Objective | Priority | How Achieved |
| --- | --- | --- |
| **Safety** | 🔴 Critical | Pressure/temperature limits enforced |
| **Product Quality** | 🟠 High | Composition control via feed ratios |
| **Production Rate** | 🟡 Medium | Feed flow controllers |
| **Energy Efficiency** | 🟢 Low | Optimize cooling water usage |

5.2 Normal Operating Conditions

| Parameter | Target Value | Tolerance | Control Action |
| --- | --- | --- | --- |
| Reactor Temperature | 125°C | ±5°C | Adjust cooling water (XMV 10) |
| Reactor Pressure | 2800 kPa | ±50 kPa | Adjust purge valve (XMV 6) |
| Reactor Level | 50% | ±10% | Adjust recycle valve (XMV 5) |
| Production Rate | 8000 kg/h | ±500 kg/h | Adjust feed flows (XMV 1-4) |
| Product G/H Ratio | 50/50 | ±5% | Adjust A/C and D/E feed ratios |

5.3 Fault Response Strategy

When a fault (IDV) is introduced:

1. Sensors detect deviation (XMEAS values change)
   ↓
2. Controllers respond (XMV values adjust)
   ↓
3. PCA detects anomaly (T² > threshold)
   ↓
4. LLM analyzes root cause
   ↓
5. Operator takes corrective action

6. Anomaly Detection System

6.1 PCA-Based Detection

Why PCA?

Reduces 52 dimensions to ~10 principal components

Captures 95%+ of process variance

Detects multivariate anomalies (not just single-variable alarms)

How It Works:

- Training Phase: Collect 100+ samples of normal operation

- Build Model: Calculate principal components from normal data

- Monitoring Phase: Project new data onto principal components

- Anomaly Score: Calculate T² statistic

- Alarm: Trigger if T² > threshold

6.2 Feature Importance

All 52 features are used

XMEAS 1-22: Basic process measurements (22 features)

XMEAS 23-41: Composition measurements (19 features) ← Critical for fault detection!

XMV 1-11: Manipulated variables (11 features) ← Shows controller response!

Why 52 features?

Composition changes are key indicators of many faults (IDV 1, 4, 8, 11)

XMV values show how controllers are responding to disturbances

More features = better fault coverage (not overfitting due to PCA dimensionality reduction)

7. System Architecture

7.1 Software Stack

┌─────────────────────────────────────────┐
│ Frontend (React + TypeScript)           │
│ - Real-time charts (Recharts)          │
│ - Mantine UI components                │
│ - Port: 5173                            │
└──────────────┬──────────────────────────┘
               ↓ HTTP/WebSocket
┌─────────────────────────────────────────┐
│ Backend API (FastAPI + Python)          │
│ - Multi-LLM integration                 │
│ - RAG knowledge base                    │
│ - Port: 8000                            │
└──────────────┬──────────────────────────┘
               ↓
┌─────────────────────────────────────────┐
│ Unified Console (Flask + Python)        │
│ - TEP simulation control               │
│ - Anomaly detection (PCA)              │
│ - Port: 9002                            │
└──────────────┬──────────────────────────┘
               ↓
┌─────────────────────────────────────────┐
│ TEP Simulation (Fortran)                │
│ - temain_mod.so (compiled binary)      │
│ - tep2py.py (Python wrapper)           │
└─────────────────────────────────────────┘

7.2 Data Flow

TEP Fortran Simulation
    ↓ (every 3 minutes)
CSV File (data/live_tep_data.csv)
    ↓ (every 6 minutes)
PCA Anomaly Detection
    ↓ (if anomaly detected)
Multi-LLM Analysis (Gemini, Claude, LMStudio)
    ↓
Markdown Reports (RCA_Results/)
    ↓
Interactive RCA Chat (with RAG context)

8. Quick Reference Tables

8.1 Control Setpoints

| SETPT # | Description | Default Value | Unit | Adjustable? |
| --- | --- | --- | --- | --- |
| SETPT(7) | Product Separator Level | 50.0 | % | ✅ Yes |
| SETPT(8) | Stripper Level | 50.0 | % | ✅ Yes |
| SETPT(10) | Reactor Cooling Water Temp | 94.6 | °C | ✅ Yes |
| SETPT(1-6, 9, 11-20) | Reserved/Unused | 0.0 | - | ❌ No |

8.2 Fault Types (IDV)

| IDV # | Description | Type | Typical Impact |
| --- | --- | --- | --- |
| IDV(1) | A/C Feed Ratio, B Composition Constant | Step | Composition shift |
| IDV(2) | B Composition, A/C Ratio Constant | Step | Composition shift |
| IDV(3) | D Feed Temperature | Step | Reactor temperature change |
| IDV(4) | Reactor Cooling Water Inlet Temp | Step | Reactor temperature rise |
| IDV(5) | Condenser Cooling Water Inlet Temp | Step | Separator pressure rise |
| IDV(6) | A Feed Loss | Step | Production rate drop |
| IDV(7) | C Header Pressure Loss | Step | Feed flow reduction |
| IDV(8) | A, B, C Feed Composition | Random | Composition variation |
| IDV(9) | D Feed Temperature | Random | Temperature variation |
| IDV(10) | C Feed Temperature | Random | Temperature variation |
| IDV(11) | Reactor Cooling Water Inlet Temp | Random | Temperature variation |
| IDV(12) | Condenser Cooling Water Inlet Temp | Random | Pressure variation |
| IDV(13) | Reaction Kinetics | Slow Drift | Gradual performance loss |
| IDV(14) | Reactor Cooling Water Valve | Sticking | Temperature control difficulty |
| IDV(15) | Condenser Cooling Water Valve | Sticking | Pressure control difficulty |
| IDV(16-20) | Unknown | Various | Research purposes |

8.3 System Timing

| Event | Interval | Configurable? |
| --- | --- | --- |
| TEP Simulation Step | 3 minutes | ❌ No (Fortran hardcoded) |
| Anomaly Detection Check | 6 minutes | ✅ Yes (2× TEP step) |
| LLM Analysis Trigger | 12 minutes | ✅ Yes (4× TEP step) |
| ULTRA START Mode | 50× speed | ✅ Yes (demo mode) |