TEP Base Case Steady-State Operating Conditions

Key Points:

Mode 1 = Base case (50% G / 50% H product split)

Production Rate: ~8000 kg/h total product

Reactor Temperature: ~120-135°C (exothermic reactions)

Reactor Pressure: ~2700-2900 kPa

All IDV = 0 (no disturbances)

**Table of Contents**

[Base Case Configuration](#1-base-case-configuration)

[Manipulated Variables (XMV) - Default Values](#2-manipulated-variables-xmv---default-values)

[Process Measurements (XMEAS) - Steady-State Values](#3-process-measurements-xmeas---steady-state-values)

[Feed Flow Rates](#4-feed-flow-rates)

[Reactor Conditions](#5-reactor-conditions)

[Product Composition](#6-product-composition)

[Control Setpoints](#7-control-setpoints)

[Operating Constraints](#8-operating-constraints)

1. Base Case Configuration

1.1 Operating Mode

Mode 1 (Base Case):

Product G/H Ratio: 50% / 50% (equal production)

Production Rate: Maximum throughput (~8000 kg/h)

G/H Mass Ratio: 50/50 (balanced production)

Other Modes (not used in this system):

Mode 2: G/H = 10/90 (high H production)

Mode 3: G/H = 90/10 (high G production)

Mode 4-6: Various production rates and ratios

1.2 Initial Conditions

All Disturbances (IDV) = 0:

IDV(1) through IDV(20) = 0.0 (no faults)

System starts from steady-state

Pre-run period: 50 steps (150 minutes) to stabilize

Simulation Parameters:

Time Step: 3 minutes per sample

Sampling Frequency: 180 seconds (3 min)

Fortran Internal Step: 1 second (for accurate physics)

2. Manipulated Variables (XMV) - Default Values

These are the baseline valve positions for steady-state operation:

| XMV # | Description | Default Value | Unit | Physical Meaning |
| --- | --- | --- | --- | --- |
| **XMV(1)** | D Feed Flow (stream 2) | **63.0** | % | D feed valve position |
| **XMV(2)** | E Feed Flow (stream 3) | **53.0** | % | E feed valve position |
| **XMV(3)** | A Feed Flow (stream 1) | **24.0** | % | A feed valve position |
| **XMV(4)** | A and C Feed Flow (stream 4) | **61.0** | % | A+C feed valve position |
| **XMV(5)** | Compressor Recycle Valve | **22.0** | % | Recycle loop valve |
| **XMV(6)** | Purge Valve (stream 9) | **40.0** | % | Purge gas valve |
| **XMV(7)** | Separator Pot Liquid Flow (stream 10) | **38.0** | % | Separator outlet valve |
| **XMV(8)** | Stripper Liquid Product Flow (stream 11) | **46.0** | % | Product outlet valve |
| **XMV(9)** | Stripper Steam Valve | **47.0** | % | Steam heating valve |
| **XMV(10)** | Reactor Cooling Water Flow | **41.0** | % | Reactor cooling valve |
| **XMV(11)** | Condenser Cooling Water Flow | **18.0** | % | Condenser cooling valve |

Important Notes:

Changing these values will disturb the process

Controllers will adjust these values to maintain setpoints

3. Process Measurements (XMEAS) - Steady-State Values

3.1 Flow Measurements (Steady-State)

| XMEAS # | Description | Steady-State Value | Unit | Typical Range |
| --- | --- | --- | --- | --- |
| XMEAS(1) | A Feed (stream 1) | ~0.25 | kscmh | 0.15-0.35 |
| XMEAS(2) | D Feed (stream 2) | ~3650 | kg/h | 3500-3800 |
| XMEAS(3) | E Feed (stream 3) | ~4500 | kg/h | 4300-4700 |
| XMEAS(4) | A and C Feed (stream 4) | ~9.0 | kscmh | 8.5-10.0 |
| XMEAS(5) | Recycle Flow (stream 8) | ~27 | kscmh | 25-29 |
| XMEAS(6) | Reactor Feed Rate (stream 6) | ~42 | kscmh | 40-45 |
| XMEAS(10) | Purge Rate (stream 9) | ~0.5 | kscmh | 0.1-2.0 |
| XMEAS(14) | Prod Sep Underflow (stream 10) | ~25 | m³/h | 20-30 |
| XMEAS(17) | Stripper Underflow (stream 11) | ~20 | m³/h | 15-25 |
| XMEAS(19) | Stripper Steam Flow | ~300 | kg/h | 200-400 |

3.2 Pressure Measurements (Steady-State)

| XMEAS # | Description | Steady-State Value | Unit | Typical Range |
| --- | --- | --- | --- | --- |
| XMEAS(7) | Reactor Pressure | **2800** | kPa | 2700-2900 |
| XMEAS(13) | Product Sep Pressure | **2700** | kPa | 2600-2800 |
| XMEAS(16) | Stripper Pressure | **3200** | kPa | 3000-3500 |

3.3 Level Measurements (Steady-State)

| XMEAS # | Description | Steady-State Value | Unit | Typical Range |
| --- | --- | --- | --- | --- |
| XMEAS(8) | Reactor Level | **50** | % | 40-70 |
| XMEAS(12) | Product Sep Level | **50** | % | 40-70 |
| XMEAS(15) | Stripper Level | **50** | % | 40-70 |

Note: All levels maintained at 50% by level controllers

3.4 Temperature Measurements (Steady-State)

| XMEAS # | Description | Steady-State Value | Unit | Typical Range |
| --- | --- | --- | --- | --- |
| XMEAS(9) | Reactor Temperature | **125** | °C | 120-135 |
| XMEAS(11) | Product Sep Temp | **80** | °C | 70-90 |
| XMEAS(18) | Stripper Temperature | **70** | °C | 60-80 |
| XMEAS(21) | Reactor Cooling Water Outlet Temp | **94.6** | °C | 85-100 |
| XMEAS(22) | Separator Cooling Water Outlet Temp | **77** | °C | 70-85 |

3.5 Other Measurements (Steady-State)

| XMEAS # | Description | Steady-State Value | Unit |
| --- | --- | --- | --- |
| XMEAS(20) | Compressor Work | **250** | kW |

4. Feed Flow Rates

4.1 Feed Streams

Stream 1 (A Feed):

Flow Rate: ~0.25 kscmh

Composition: Pure A

Temperature: Ambient

Pressure: Feed header pressure

Stream 2 (D Feed):

Flow Rate: ~3650 kg/h

Composition: Pure D

Temperature: Variable (IDV_3 can disturb)

Pressure: Feed header pressure

Stream 3 (E Feed):

Flow Rate: ~4500 kg/h

Composition: Pure E

Temperature: Ambient

Pressure: Feed header pressure

Stream 4 (A and C Feed):

Flow Rate: ~9.0 kscmh

Composition: A + C mixture (- Component A: ~40 mole %; - Component C: ~60 mole %)

Temperature: Ambient

Pressure: Feed header pressure

4.2 Total Feed Rate

Total Mass Flow to Reactor: ~8000 kg/h (approximate)

Molar Ratios (for Mode 1):

Designed to produce 50% G and 50% H

Stoichiometric balance for reactions:

- A + C + D → G

- A + C + E → H

5. Reactor Conditions

5.1 Reactor Operating Point

| Parameter | Steady-State Value | Unit | Control Method |
| --- | --- | --- | --- |
| **Temperature** | **125** | °C | Cooling water flow (XMV 10) |
| **Pressure** | **2800** | kPa | Purge valve (XMV 6) |
| **Level** | **50** | % | Compressor recycle (XMV 5) |
| **Feed Rate** | **42** | kscmh | Feed valves (XMV 1-4) |

5.2 Reactor Type and Characteristics

Reactor Type: CSTR (Continuous Stirred Tank Reactor)

Key Characteristics:

Exothermic Reactions: Heat generation requires cooling

Gas-Phase Reactions: Reactants A, C, D, E are gases

Liquid-Phase Products: Products G, H, F are liquids

Well-Mixed: Uniform composition and temperature

Agitator Speed: Controlled (XMV 12, not shown in standard 11 XMV)

5.3 Reaction Kinetics

Primary Reactions:

A(g) + C(g) + D(g) → G(liq)    (Product G)
A(g) + C(g) + E(g) → H(liq)    (Product H)

Side Reactions:

A(g) + E(g) → F(liq)           (Byproduct F)
3D(g) → 2F(liq)                (Side reaction)

Reaction Rates:

Temperature-dependent (Arrhenius equation)

Pressure-dependent (gas-phase reactions)

Composition-dependent (reactant concentrations)

6. Product Composition

6.1 Product Stream (Stream 11 - Stripper Underflow)

Mode 1 Target Composition:

| Component | Target Mole % | Typical Range |
| --- | --- | --- |
| **G (Product)** | **50** | 48-52 |
| **H (Product)** | **50** | 48-52 |
| **D** | <1 | 0-2 |
| **E** | <1 | 0-2 |
| **F (Byproduct)** | <5 | 2-8 |

Total Product Rate: ~8000 kg/h (G + H + F)

Product Quality Metrics:

G/H Ratio: 50/50 (Mode 1 specification)

Purity: >95% (G + H combined)

Byproduct F: <5% (minimize)

6.2 Purge Stream (Stream 9)

Composition (approximate):

Component A: ~30 mole %

Component B: ~10 mole % (inert)

Component C: ~30 mole %

Component D: ~5 mole %

Component E: ~5 mole %

Component F: ~5 mole %

Component G: ~10 mole %

Component H: ~5 mole %

Purpose: Remove inerts and prevent buildup

7. Control Setpoints

7.1 Active Setpoints (Mode 1)

| SETPT # | Description | Default Value | Unit | Controller |
| --- | --- | --- | --- | --- |
| **SETPT(7)** | Product Separator Level | **50.0** | % | LC-7 |
| **SETPT(8)** | Stripper Level | **50.0** | % | LC-8 |
| **SETPT(10)** | Reactor Cooling Water Outlet Temp | **94.6** | °C | TC-10 (Cascade) |

8. Operating Constraints

8.1 Safety Limits

| Parameter | Normal Range | Critical Limit | Consequence if Exceeded |
| --- | --- | --- | --- |
| **Reactor Temperature** | 120-135°C | **Max: 140°C** | Thermal runaway risk |
| **Reactor Pressure** | 2700-2900 kPa | **Max: 3000 kPa** | Vessel rupture risk |
| **Reactor Level** | 40-70% | **Min: 30%, Max: 80%** | Pump cavitation / Overflow |
| **Compressor Work** | 200-300 kW | **Max: 400 kW** | Compressor overload |

8.2 Product Quality Constraints

| Specification | Target | Acceptable Range |
| --- | --- | --- |
| **G/H Ratio** | 50/50 | 48/52 to 52/48 |
| **Product Purity** | >95% | >93% |
| **Byproduct F** | <5% | <8% |

8.3 Environmental Constraints

| Parameter | Limit | Reason |
| --- | --- | --- |
| **Purge Rate** | <2.0 kscmh | Emissions limit |
| **Cooling Water Temp** | <105°C | Thermal pollution |

9. How to Achieve Steady-State

9.1 Startup Procedure

- Initialize XMV Values: Set all XMV to default values (see Section 2)

- Set IDV = 0: Ensure no disturbances active

- Pre-Run Period: Run simulation for 50 steps (150 minutes) with IDV = 0

- Check Stability: Verify all XMEAS values are within normal ranges

- Enable Controllers: Activate all 11 control loops

- Monitor: Observe for 1-2 hours to confirm steady-state

9.2 Steady-State Verification

Criteria for Steady-State:

All XMEAS values stable (< 1% change over 30 minutes)

All XMV values stable (< 0.5% change over 30 minutes)

Reactor temperature within 120-135°C

Reactor pressure within 2700-2900 kPa

Product composition G/H ratio = 50/50 ± 2%

9.3 Common Issues

Problem: Reactor temperature rising

Cause: Insufficient cooling water flow

Solution: Increase XMV(10) or decrease feed rates

Problem: Reactor pressure rising

Cause: Insufficient purge rate

Solution: Increase XMV(6) (purge valve)

Problem: Product composition off-spec

Cause: Feed ratio imbalance

Solution: Adjust XMV(1-4) to correct A/C/D/E ratios

10. Quick Reference Table

10.1 Critical Parameters Summary

| Category | Parameter | Value | Unit |
| --- | --- | --- | --- |
| **Reactor** | Temperature | 125 | °C |
|  | Pressure | 2800 | kPa |
|  | Level | 50 | % |
| **Feeds** | D Feed | 3650 | kg/h |
|  | E Feed | 4500 | kg/h |
|  | A Feed | 0.25 | kscmh |
|  | A+C Feed | 9.0 | kscmh |
| **Product** | Total Rate | 8000 | kg/h |
|  | G/H Ratio | 50/50 | % |
| **Cooling** | Reactor Cooling Water Outlet | 94.6 | °C |
|  | Reactor Cooling Water Flow | 41% | % |